# Paclink…a Modern AX.25 Packet Client

# Compatible with Outlook Express

Rick Muething, KN6KB   mailto:rmuething@cfl.rr.com

Vic Poor, W5SMM           mailto:vpoor@cfl.rr.com

# Overview:

Paclink is a new implementation of a flexible, easily configured personal BBS (radio E-mail) client that interfaces with most popular ham BBS systems and most popular E-mail client programs such as Microsoft Outlook Express.  Essentially Paclink adds packet radio and Telnet channels to Outlook Express.  Paclink is compatible with the Winlink 2000 system (including B2F protocol which supports attachments) and popular FBB compatible BBSs.  Paclink uses the AGW Packet Engine to support a large number of TNCs and sharing TNCs across applications. Figure 1 shows the interfaces between the Paclink program and the other subsystems that complete the Personal BBS client.

Figure 1.  Paclink Radio Client and Subsystem Interfaces. The Pactor TNC is required only if HF Pactor capability is needed.

One implementation feature is that Paclink uses standardized TCP or Telnet protocols for communicating between all major subsystems.  The TCP connections are normally on the same computer, but could be located remotely on a LAN or accessed via the Internet. This allows, for example, accessing Paclink remotely over the Internet using any standard POP3/SMTP client with the correct user and password settings.

Another significant aspect of the Paclink architecture is its use of existing POP3 and SMTP compatible E-mail clients to provide much of the user interface, message addressing, composition, management and display functions.  This is very important in emergency applications where keeping user interfaces familiar is critical.

Paclink uses a one-to-one relationship between E-mail client “accounts” and connection “channels”.  Most E-mail programs support multiple E-mail accounts and these accounts, along with the corresponding setup in Paclink specify everything necessary to establish the remote connection.   This information identifies both outgoing and incoming connections and keeps data streams and messages separate from other connections or accounts. Figure 2 illustrates how this works in Paclink.

Fig 2.  Paclink’s one-to-one relationship of channels to E-mail accounts

In figure 2 each unique E-mail account, as set up on the E-mail client, has a one-to-one association to a Paclink “channel” of the same name. This channel is then set up in Paclink to provide all necessary information to complete a radio or Telnet connection.  Channel setup includes local and remote call signs, port numbers, protocols, polling interval and any optional connection script. Once these channels are configured in Paclink, all user interaction is normally done from the E-mail client program using appropriate E-mail or Radio address conventions.

Fig 3 shows all that’s required to configure a typical packet channel in Paclink.

Figure 3.  Paclink channel and port properties for a Packet channel

After configuring each desired channel the only remaining setup required to operate Paclink is initializing the AGW Packet engine for your specific modem and radios and populating your favorite E-mail client address book with any required radio and E-mail addresses.  A simple address syntax compatible with all E-mail clients is used to differentiate radio addresses from standard E-mail. This insures proper radio addressing including conventional packet Hroutes when required. In addition to its automated BBS <> BBS forwarding protocols, Paclink also supports classical terminal mode operation when necessary to work in conversational keyboard mode.

Summary:

Paclink is a new modern tool that gives AX.25 packet and Telnet capability to modern E-mail clients. It should find many applications in Ham radio BBS and emergency messaging systems. Paclink is written in VB .NET for Windows 98 and later and requires the Microsoft .NET framework.  Paclink is free for amateur radio use. Commercial applications should contact the authors.

References:

[1] Winlink 2000…A Global Ham Message Transfer and Delivery Network;

Rick Muething, Vic Poor, Hans Kessler, and Steve Waterman. 19th ARRL and TAPR Digital Communications Conference, Sept 2000.

http://www.winlink.org/

[2] Telpac and Paclink--Streamlined AX.25 Packet Server and Client for a full service Ham Radio Messaging Network. 22nd  ARRL and TAPR Digital Communications Conference, Sept 2003

[3] The AGW Packet Engine by George Rossopoulos, SV2AGW http://www.raag.org/sv2agw/pepro.htm