URONODE(8)		 Linux System Managers Manual		    URONODE(8)

NAME

URONode - Node front end for AX.25, NET/ROM, Rose and TCP

SYNOPSIS

node

DESCRIPTION

URONode	is  a simple node front end, modelled after the node shells of

LinuxNode, AWZNode, and Flexnode.

COMMANDS

The following commands are supported for users of URONode:

?	     Give short list of available commands.

Bye	     Disconnect user from the node.

Connect <call | alias> [s|d]		     For NETROM

Connect <port> <call> [via <call1> ...] [d|s] For AX.25

Connect <call> <address> [<digi>] [d|s]	     For ROSE

Connect <destination> [s|d]		     For FlexNet

Initiates an AX25, NET/ROM, ROSE or Flexnet connection to

a	remote	host.  If more than two parameters are entered

and the second parameter is ten charachers in length then

it	 is  interpreted  as  a ROSE connection, otherwise the

first parameter is interpreted as a port name and AX25 is

used  to  make  the connection via that port. If only one

parameter is given the connection is made	searching  for

the  callsign  in	NET/ROM nodes, in AX25 Fixed links, in

FlexNet destinations and last in Mheard database in  this

order.

For  a ROSE connection the <address> part must be exactly

six or ten digits.	 If only six digits are supplied,  the

DNIC  (first  four digits) default to the local DNIC. The

local DNIC is assumed to be that of the first  configured

Rose port in /etc/ax25/rsports.

The  user	may  optionally	 supply as the last argument a

single character which modifies the default behaviour  on

disconnection  of	the  connection.   If  a single `s' is

entered as the last argument, then when the  remote  host

disconnects  you will be returned to this node. If a sin-

gle `d' is entered as the last argument, you will be dis-

connected from this node too. The Default behaviour (nei-

ther `s' nor `d' entered) is configured in the node  con-

figuration file and depends on the sysop preference.

CAllbook <call>[@<server>]

Retrieve  data about a callsign from an internet-accessi-

ble callbook server. The server name can usually be omit-

ted,  since the default server configured by the operator

is usually valid. The default setting can	be  overridden

by specifying a host name.

Escape [<escape string>]

Override  the  sysop  configured default escape character

setting. If the Escape command is given without an	 argu-

ment   then  the  current	escape	character  setting  is

returned to the user. The escape string may be  specified

using any of the well known codings:

<char>  to enter the escape character in its binary form.

^C to enter the escape character as a  control  character

value.

NNN to set the escape character to a Decimal value.

0xNN  to set the escape character to a HexaDecimal value.

0NNN to set the escape character to an Octal value.

off to disable the escape character.

Finger [<username>][@<hostname>]

Retrieve information about users of a system. If the user

name  is omitted, shows the users currently logged on the

host. If the hostname is omitted, defaults to  the	 local

host.

Help [<command>]

Give  help	 for  the specified command or this text if no

command is specified. Commands can	 not  be  abbreviated.

Use  the "?" command to retrieve a list of available com-

mands.

HOst <hostname> | <ip address>

Give the Domain Name Service host name information	 about

<hostname> or <ip address>.

Info	     Display  the  version information and the contents of the

/etc/ax25/node.info  file,	 which	should	describe   any

aspects of your system that you would like to brag about.

INTerfaces    Display the node's interface  schema,  most  folks	 often

call these "ports".

Links [d | n | v | <call>]

Shows the AX25 link table of the local node. The destina-

tions on this list can be reached using the Connect  com-

mand without knowing the actual network path used (assum-

ing the network is OK).

The optional parameter d show  the	 AX25  nodes  directly

linked.   The  optional  parameter	 n show the AX25 nodes

linked connecting another node and passing a string  like

"C	 XX0XX".  The optional parameter v show the AX25 links

via digipeaters.

You can also specify a destination callsign  to  get  the

verbose information for a single destination.

Mheard [<portname>]

Give  a  list  of	heard  AX.25 stations on the specified

port.

Nodes [* | <node>]

Show the NET/ROM node table of the local host. The	 nodes

on	 this  list  can  be reached using the Connect command

without knowing the actual network	 path  used  (assuming

the network is OK).

The  optional  argument '*' toggles verbose mode, showing

the Obsolescence counter, relative path quality  and  the

port  and neighbour node used to reach each node. You can

also specify a node callsign to get the verbose  informa-

tion  for	a  single  node.  In that case a "which" field

that tells what route the kernel will use	to  reach  the

node is also shown.

Ping <host> [<size>]

Check  if	a  host	 can  be reached trough the network by

sending an ICMP Echo Request packet to the host and wait-

ing  for  it  to reply. If a reply is received the round-

trip-time (RTT) between the local	and  remote  hosts  is

shown.

If	 an  optional  length is specified the data portion of

the packet is filled with length number of bytes.

Routes	     Shows the NET/ROM route table of the local host (eg.  the

nodes  which  the	local node directly talks with). These

nodes are used to reach the other nodes on the  node  ta-

ble. Fields shown are:

Link Is there an AX25 connection active to this node.

Port Which port is this route on.

Callsign The callsign of the neighbour node.

Quality A relative quality for the path (0-255).

Destinations  Number  of  other  nodes  reached  via this

route.

Lock Is the quality of this route locked by the operator.

QSO Number of NET/ROM session enties.

Telnet <host> [<port>] [<string>] [d|s]

Initiate  a telnet session to a remote host using TCP/IP.

By default, the telnet command connects to the  TCP  port

23	 (allocated  for  telnet). You can specify another TCP

port or a TCP port name.

If an optional third argument  <string>  is  given,  that

string,  followed	by  a  CRLF is sent to the remote host

right after the connection is established. This is mainly

useful for command aliases.

If	 a  single  `s' is entered as the last parameter, then

when the remote host disconnects you will be returned  to

this node.	 If a single `d' is entered as the last param-

eter, you	will  be  disconnected	from  this  node  too.

Default  behaviour	 (neither `s' nor `d' entered) depends

on sysop configuration.

MSG <user> <message>

Send a message to another user of the node. The  user  in

question  must  be	 in idle state (ie. not connected/con-

necting anywhere or running a program).

If the user has an SSID other than zero, the SSID must be

specified.	 If multiple users are logged in with the same

callsign/SSID pair, those who are in idle state, get  the

message.

Users	     Show  a  list  of	users currently connected to the local

node, where the users are coming from, and what are  they

doing at the moment.

FILES

/etc/ax25/uronode.conf

URONode configuration file.

/etc/ax25/uronode.perms

URONode permissions file.

/etc/ax25/uronode.users

URONode users settings file.

/etc/ax25/uronode.motd

URONode message of the day file.

/etc/ax25/uronode.info

The response to the 'info' command.	 This file should be edited to

reflect the local configuration.

/var/ax25/node/loggedin

Database of current users.

/var/ax25/mheard/mheard.dat

Information about AX.25 stations heard.

/usr/lib/ax25/node/help/*.hlp

The online help files.

SEE ALSO

uronode.conf(5), uronode.perms(5), axports(5), ax25d(8), mheardd(8).

AUTHOR

Brian Rogers N1URO <n1uro@n1uro.com>

THANKS TO

Stefano Noferi IZ5AWZ <iz5awz@radio-gw.cnuce.cnr.it>

Tomi Manninen OH2BNS <tpmannin@cc.hut.fi>

Alan Cox GW4PTS <gw4pts@gw4pts.ampr.org>

Roy PE1RJA <roy@esrac.ele.tue.nl>

Linux				 28 April 1998			    URONODE(8)